Name ____________________

School District ___________________

ELDT Theory Test

Participants must receive an 80% to pass this course

- T    F   Pre-trip must be done each morning before the driver takes control of the

bus.

- T   F    Under the hood items must be checked at least once in a 24-hour period

when a bus is being operated to transport students.

- T   F   On hydraulic brake system, you check the brake motor by pressing the

petal until you hear the motor noise, while the engine is running.

- T   F   To check the neutral safety switch on the bus, put the bus in any gear

except neutral and try to start the bus.

- T   F   Never use your mirrors to check your 8-way lights.

- T   F   To check the parking brake, apply light pressure to the accelerator the bus

should not move forward.

- T   F   The State curriculum say a 20-inch steering wheel should not move more

than 25 degrees.

- T   F   In post trip inspection you should; check for any sleeping children, items

left on the bus, sweep the bus if necessary, and record any problems.

- T   F   On your way to the back of the bus, pull on all the seats to check for

security.

- T   F   All roof hatches do not need warning buzzers.

- T   F   Look under the bus for leaks and anything broken, bent, or hanging down.

- T   F   Tread depth on the front tires should be at least 8/32 to be safe.

- T   F   Check to make sure that the exhaust is not protruding more than 6 inches.

- T   F  The four way flashers must be turned on within 200 feet of railroad tracks

in rural areas.

- T   F   As a driver, you should always be searching for distracted drivers and

pedestrians.

- T   F   It is acceptable to use your cell phone for personal use while driving a

school bus.

- T   F  When following another vehicle, you should always follow the 2 second

rule.

- T   F  The best practice while driving in inclement weather is to reduce speed.

- T   F   When stopping at railroad crossings, you should stop no closer than 3 feet

and no farther than 100 feet.

- T   F   When stopped at a railroad crossing you should open service door, open

driver window, and eliminate all interior noise.

- T   F   Daily maintenance checks include: oil, power steering, brake fluid, and

coolant.

- T   F   It is important for a bus driver to get a full nights sleep and be mentally

and physically prepared for the day.

- T   F   Driver must be drug tested after an accident if: someone is injured and

must be transported by ambulance, one or more vehicles towed, or

a citation is issued.

- T   F   A bus driver cannot question the safety practices of an employer without

incurring the risk of losing a job or being subject to reprisals simply for

stating a safety concern.

- T   F  You must have a blood alcohol under .025 in order to operate a school

bus.

- T   F   You should always evacuate all students after a bus crash occurs.

- T   F   A bus driver should train students at least once a year in evacuation

procedures.

- T   F   Never use your crossover mirrors for viewing traffic.

- T   F   Students can remain on the bus while fueling.

- T   F   It is permissible to block the rear exit with sports equipment.

- T   F   Students are permitted to proceed to the front of the bus while the bus is

slowing down.

- T   F   Busses should stop at all state and federal weigh stations.

- T   F   Driver is authorized to assign seats.

- T   F   Yelling at students is an acceptable student management technique.

- T   F   Mirrors should be adjusted anytime a new driver takes control of the bus.

- T   F   The most dangerous time for a student is during the loading and

unloading procedures.

- T   F   A driver should visually inspect: stop arms, emergency exits, fire

extinguishers, passenger seats, first aid kits, interior lights, and

temperature controls.

- T   F   Pre-trip and post-trip inspections must be in writing and turned in to the

office for filing.

- T   F  It is permissible for the driver to change the route without consulting the

transportation director.

- T   F   Driver is not required to report suspicious activity on his or her route.

Answer Key

- T						36.   T

- T						37.   T

- F						38.   F

- T                                                              39.   F

- F                                                              40.   F

- T

- F

- T

- T

- F

- T

- F

- F

- T

- T

- F

- F

- T

- F

- T

- T

- T

- T

- F

- F

- F

- T

- T

- F

- F

- F

- F

- T

- F

- T